An effective way to test the health of a battery pack is to place a normal operating load on a fully charged pack, and then record the voltage levels while it is discharging through a test load. The information produced from the recorded data will create a graphical “discharge curve” that can be used to examine its performance level and determine the health of the battery under test.

I built an aircraft battery tester using the parallel port on an
old 386 Toshiba (pre-Windoz) laptop to control and log the
discharge/charge cycle test. The purpose is determine the capacity
of the battery to see if it should be replaced or not. (FAA mandated
testing for continued airworthiness of said batteries).

The test code is written in QuickBasic. It logs the discharge/charge
curves vs hours, integrates the curves to compute A-H capacity, etc. The
bits on the parallel port turn on/off the load, turn on/off the charger,
and clock a serial A/D converter with records the battery voltage.

I used some giant electric locomotive braking resistors as the loads.
The load is sized to discharge a 12V battery at 35A, meaning that the
power dissipation is 420Watts (no fan). see:
http://www.meci.com/default.asp?mode...s&category=150

I used an automotive starter relay (rated at 1000A) to switch the load
on and off. A power NFET is used to switch the relay coil on/off using
one of the parallel port bits. The software monitors the discharging
battery, and disconnects the load when the battery voltage reaches 11.5V.

I also have a 2A constant current charger for recharging the battery as
part of the test. The charger is controlled using an isolated AC switch
module. The software cut off the charger when the battery reaches 14.5V.

I used a 12bit A/D converter to measure battery voltage during
charge/discharge/resting. It is interfaced to the parallel port using
three pins: StartConvert, Serial clock, and Data... 12 bits lets me
record the voltage to about 3 1/2 significant figures (i.e. 12.85V)

I capture the charge/discharge curves in a comma-separted text file
I move the file to a windows PC running EXCEL. Makes really nice plots.
Net output is the battery capacity expressed in Ampere-Hours.

MikeM
Skylane '1MM
Pacer '00Z