Lead Acid BATTERIES

LOAD – 12V, 20W lamp

Used for 6 – 12V batteries

Typical discharge current is 1.6Amps for a 12V battery and 1Amp for a 6V battery.

Due to the internal shunt resistor the circuit is limited to 2Amps maximum.

Capacities range from 1200mAh to >10Ah

A 10Ah capacity battery will take >6 hours to test.

NiCd & NimH BATTERIES

LOAD – 1.2R ( Plus 0.1R shunt resistor )

Used for 1.2 – 1.5V batteries.

Typical discharge current is 0.9Amps for a NiCd or NimH

AAA, AA capacities range from 600mAh to 2600mAh

Discharge time at 600mAh = 0.7 hours

Discharge time at 2600mAh = 2.9 hours

LOAD – 0.25R ( Plus 0.1R shunt resistor )

D cells have ratings up to 11,000mAh

Typical discharge current is 3.4Amps for a NiCd or NimH

Discharge time at 11,000mAh = 3.25 hours

LOAD – 40R

Used for 9V batteries

Typical discharge current is 0.23Amps for a NiCd or NimH

PP3 capacities range from 100mAh to 300mAh

Discharge time at 100mAh = 0.4 hours

Discharge time at 300mAh = 1.3 hours